| Driver Version | Description | Author |
| --- | --- | --- |
| v1.2 | Initial version of document | TDK Invensense JW Yoon |
| v1.4 | Remove functions related to ‘SAVE_ALL_CONFIG’ register. | TDK Invensense JW Yoon |
| v1.5 | Remove a function related to ‘SELECT STREAMING INTERFACE’ command. | TDK Invensense JW Yoon |
| V2.0 | Add support for ENABLE(DISABLE)_SENSORFT' commands. Add support for Sensitivity and Misalignment correction. Add support for Firmware update. | TDK Invensense JW Yoon |

- Key APIs Notes

void IIM4623x_set_serif

Function:

Set serial interface to send command to device and receive response from device.

Parameters:

uint32_t (*rx)(uint8_t *, uint32_t),

The function pointer of Rx API depending on platform system via SPI or UART.

uint32_t (*tx)(uint8_t *, uint32_t),

The function pointer of Tx API depending on platform system via SPI or UART.

uint32_t (*tx_rx)(uint8_t *, uint32_t, uint8_t *, uint32_t)

The function pointer of Tx/Rx API depending on platform system via SPI or UART.

Return:

NULL.

void IIM4623x_set_reset

Function:

Set serial interface to reset device.

Parameters:

void (*reset)(void)

The function pointer of Reset API depending on platform system.

Return:

NULL.

uint32_t IIM4623x_ serif_rx

Function:

Wrapper function of Rx API to receive response from device.

Parameters:

uint8_t *buf,

Pointer to packet buffer for Rx.

uint32_t len

Size of buffer.

Return:

0: Success, Others: Fail.

uint32_t IIM4623x_ serif_tx

Function:

Wrapper function of Tx API to send command to device.

Parameters:

uint8_t *buf,

Pointer to packet buffer for Tx.

uint32_t len

Size of buffer.

Return:

0: Success, Others: Fail.

uint32_t IIM4623x_ serif_tx_rx

Function:

Wrapper function of Tx/Rx API to send command to device and receive response from device.

Parameters:

uint8_t *wbuf,

Pointer to packet buffer for Tx.

uint32_t wlen,

Size of Tx buffer.

uint8_t *rbuf,

Pointer to packet buffer for Rx.

uint32_t rlen

Size of Rx buffer.

Return:

0: Success, Others: Fail.

void IIM4623x_Reset

Function:

Wrapper function of Reset API.

Parameters:

NULL.

Return:

NULL.

PartNum IIM4623x_Get_PartNum

Function:

Return part number.

Parameters:

NULL.

Return:

0: IIM46230, 1: IIM46234.

void IIM4623x_SetCMD_Firmware

Function:

Set command packet for firmware update.

Parameters:

uint8_t cmd_type

Type of command.

Return:

NULL.

void IIM4623x_Switch_to_Bootloader

Function:

Set UPGRADE_REQUEST flag in the flash.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Upgrade_Firmware

Function:

Bootloader prepares Firmware upgrade.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Send_Length_Info

Function:

Send information on length of firmware image to bootloader.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Send_Image_Data

Function:

Send packet containing firmware image data to bootloader.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Clear_Upgrade_Flag

Function:

Clear UPGRADE_REQUEST flag in the flash.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Initialize

Function:

Initialize variables for driver status. Reset device. Check WHO_AM_I.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_SetCMD_Common

Function:

Set packet for common command.

Parameters:

uint8_t cmd_type

Type of command.

Return:

NULL.

void IIM4623x_SetCMD_ReadRegister

Function:

Set command packet to read register.

Parameters:

reg user_reg

User register.

Return:

NULL.

void IIM4623x_ SetCMD_WriteRegister

Function:

Set command packet to write register.

Parameters:

reg user_reg,

User register.

uint8_t *value

Pointer to value to be written to register.

Return:

NULL.

void IIM4623x_Set_UtcTime

Function:

Set command packet to set UTC time.

Parameters:

utc time

UTC time to be set.

Return:

NULL.

void IIM4623x_Read_WhoAmI

Function:

Set and send command packet to read WHO_AM_I from register.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Get_SerialNum

Function:

Set and send command packet to read serial number from register.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Get_Version

Function:

Set and send command packet to read firmware version from register.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Imu_SelfTest

Function:

Set and send command packet to invoke self-test.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Set_OutDataForm

Function:

Set and send command packet to set output data form.

Parameters:

enum IIM4623x_OutDataForm form

Output data form to be set.

(0: 32-Bit IEEE 754 single-precision floating point,

1: 32-Bit Fixed point 2's Complement representation).

Return:

NULL.

void IIM4623x_Set_SampleRateDiv

Function:

Set and send command packet to set sample rate divisor.

Parameters:

enum IIM4623x_SampleRateDiv divisor

Sample rate divisor for each ODR (1 ~ 100).

Return:

NULL.

void IIM4623x_Toggle_DataOutput

Function:

Set and send command packet to toggle data output.

Parameters:

enum IIM4623x_DataOutPut output

0: Accel, 1: Gyro, 2: Temperature, 3: Delta Velocity, 4: Delta Angle.

Return:

NULL.

void IIM4623x_Set_UartIfConfig

Function:

Set and send command packet to set UART baud rate.

Parameters:

enum IIM4623x_UartBaudRate baud

0: 921600, 1: 1500000, 2: 2000000, 3: 3000000.

Return:

NULL.

void IIM4623x_Set_SyncConfig

Function:

Set and send command packet to enable/disable PPS interface.

Parameters:

enum IIM4623x_SyncConfig sync_mode

0: Disable PPS, 1: Enable PPS.

Return:

NULL.

void IIM4623x_Set_BWConfig_Accel

Function:

Set and send command packet to set LPF BW of Accel.

Parameters:

enum IIM4623x_AccBwConfig bw

0x40: BW4, 0x50: BW5, 0x60: BW6, 0x70: BW7.

Return:

NULL.

void IIM4623x_Set_BWConfig_Gyro

Function:

Set and send command packet to set LPF BW of Gyro.

Parameters:

enum IIM4623x_GyroBwConfig bw

0x4: BW4, 0x5: BW5, 0x6: BW6, 0x7: BW7.

Return:

NULL.

void IIM4623x_Set_AccelConfig0

Function:

Set and send command packet to set FSR of Accel.

Parameters:

enum IIM4623x_AccelConfig0 fsr

0x00: 16G, 0x20: 8G, 0x40: 4G, 0x60: 2G.

Return:

NULL.

void IIM4623x_Set_GyroConfig0

Function:

Set and send command packet to set FSR of Gyro.

Parameters:

enum IIM4623x_GyroConfig0 fsr

0x00: 2000 DPS, 0x20: 1000 DPS, 0x40: 500(480) DPS, 0x60: 2000 DPS.

Return:

NULL.

void IIM4623x_Toggle_ExtCalibConfig

Function:

Set and send command packet to enable/disable bias & sensitivity calibration.

Parameters:

enum IIM4623x_CalibConfig config

Decide whether bias & sensitivity calibration for Accel and Gyro is enabled or not.

Return:

NULL.

void IIM4623x_Write_Bias

Function:

Set and send command packet to write bias to register.

Parameters:

enum IIM4623x_Axis axis,

0: Accel X, 1: Accel Y, 2: Accel Z, 3: Gyro X, 4: Gyro Y, 5: Gyro Z.

float bias_val

Bias value to be written.

Return:

NULL.

void IIM4623x_Read_Bias

Function:

Set and send command packet to read bias from register.

Parameters:

enum IIM4623x_Axis axis,

0: Accel X, 1: Accel Y, 2: Accel Z, 3: Gyro X, 4: Gyro Y, 5: Gyro Z.

Return:

NULL.

void IIM4623x_Write_Sens_Mat_Accel

Function:

Set and send command packet to write sensitivity matrix to register.

Parameters:

enum IIM4623x_Mat_Index indx

Index of sensitivity matrix.

float value

Value to be written to sensitivity matrix register.

Return:

NULL.

void IIM4623x_Write_Sens_Mat_Gyro

Function:

Set and send command packet to write sensitivity matrix to register.

Parameters:

enum IIM4623x_Mat_Index indx

Index of sensitivity matrix.

float value

Value to be written to sensitivity matrix register.

Return:

NULL.

void IIM4623x_Read_Sens_Mat_Accel

Function:

Set and send command packet to read sensitivity matrix from register.

Parameters:

enum IIM4623x_Mat_Index indx

Index of sensitivity matrix.

Return:

NULL.

void IIM4623x_Read_Sens_Mat_Gyro

Function:

Set and send command packet to read sensitivity matrix from register.

Parameters:

enum IIM4623x_Mat_Index indx

Index of sensitivity matrix.

Return:

NULL.

void IIM4623x_Start_Streaming

Function:

Set and send command packet to start data streaming.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Stop_Streaming

Function:

Set and send command packet to stop data streaming.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Set_SensorFT

Function:

Set and send command packet to enable/disable the sensorFT™ feature.

Parameters:

bool on_off

Decide whether the sensorFT™ feature is enabled or not.

Return:

NULL.

Uint32_t IIM4623x_Read_DataOutput

Function:

Parse packet of data stream and read output data when data ready interrupt happens.

Parameters:

NULL.

Return:

0: Success.  Others: Fail.

bool Is_Command_Mode

Function:

Check whether current mode is command mode or not.

Parameters:

NULL.

Return:

true: command mode.  false: streaming mode.

void IIM4623x_Set_StopStreaming

Function:

Set status variable which indicates that ‘STOP STREAMING’ command will be sent.

Parameters:

NULL.

Return:

NULL.

void IIM4623x_Set_StartStreaming

Function:

Set status variable which indicates that ‘START STREAMING’ command will be sent.

Parameters:

NULL.

Return:

NULL.

Uint32_t response_handler

Function:

Handle all responses from device when data ready interrupt happens.

Parameters:

NULL.

Return:

0: Success.  Others: Fail.

- External Global Variables

| Type | Name | Description |
| --- | --- | --- |
| volatile int | drdy_interrupt | Indicate that data ready interrupt happened. |
| volatile uint64_t | timestamp | Timestamp of host system |
| int | bus_type | 0: SPI, 1: UART |
| uint32_t | g_ul_buff_size | Size of buffer which is used to receive UART data in UART handler |
| volatile uint8_t* | p_revdata | Pointer to buffer which is used to receive UART data in UART handler |

- Sample Code for customer

Main function in example application int main(void) { uint8_t uc_key = 0; uint8_t uc_pre_key = 0; drdy_interrupt = 0; if (bus_type == 0) { IIM4623x_set_serif(spi_master_read, spi_master_write, spi_master_write_read); } else if (bus_type == 1) { IIM4623x_set_serif(uart_receive, uart_transmit, uart_rx_tx); } IIM4623x_set_reset(device_reset); IIM4623x_Initialize(); while (1) { switch(uc_pre_key) { case 'f': select_data_output_form(uc_key); break; case 'b': select_uart_baud_rate(uc_key); break; case 'y': select_sync_config(uc_key); break; case 'k': save_all_config(uc_key); break; case 'l': select_lpf_bw_accel(uc_key); break; case 'm': select_lpf_bw_gyro(uc_key); break; case 'a': select_fsr_accel(uc_key); break; case 'g': select_fsr_gyro(uc_key); break; case 'c': select_ext_calib_config(uc_key); break; case 'e': set_bias(uc_key); break; case 'u': read_bias(uc_key); break; case 'd': select_data_output(uc_key); break; case 'o': select_odr(uc_key); break; default: char_input_handler(uc_key); break; } } uc_pre_key = uc_key; if (drdy_interrupt == 1) { drdy_interrupt = 0; if (response_handler()) { break; } } }

Select form of data output void select_data_output_form(uint8_t input) { if (Is_Command_Mode()) { switch (input) { case '0': IIM4623x_Set_OutDataForm(FLOATING); break; case '1': IIM4623x_Set_OutDataForm(FIXED); break; default: break; } } }

Select ODR void select_odr(uint8_t input) { if (Is_Command_Mode()) { switch (input) { case '0': IIM4623x_Set_SampleRateDiv(ODR_10HZ); break; case '1': IIM4623x_Set_SampleRateDiv(ODR_20HZ); break; case '2': IIM4623x_Set_SampleRateDiv(ODR_25HZ); break; case '3': IIM4623x_Set_SampleRateDiv(ODR_50HZ); break; case '4': IIM4623x_Set_SampleRateDiv(ODR_100HZ); break; case '5': IIM4623x_Set_SampleRateDiv(ODR_125HZ); break; case '6': IIM4623x_Set_SampleRateDiv(ODR_200HZ); break; case '7': IIM4623x_Set_SampleRateDiv(ODR_250HZ); break; case '8': IIM4623x_Set_SampleRateDiv(ODR_500HZ); break; case '9': IIM4623x_Set_SampleRateDiv(ODR_1KHZ); break; default: break; } } }

Select streaming data output void select_data_output(uint8_t input) { if (Is_Command_Mode()) { switch (input) { case '0': IIM4623x_Toggle_DataOutput(ACCEL); break; case '1': IIM4623x_Toggle_DataOutput(GYRO); break; case '2': IIM4623x_Toggle_DataOutput(TEMP); break; case '3': IIM4623x_Toggle_DataOutput(DELTA_VEL); break; case '4': IIM4623x_Toggle_DataOutput(DELTA_ANG); break; default: break; } } }