- Introduction

The purpose of this document is to give a guide on the use of the IIM4623x driver SW and example application based on Atmel SAMV71 platform. The IIM-4623x is a 6-axis Motion Tracking device that combines multiple 3-axis gyroscopes and 3-axis accelerometers. The IIM-4623x includes multiple capabilities to enable easy, robust and accurate inertial measurements in Industrial applications.

- HW Requirement

2.1. HW boards

To test the driver SW and example application, the following 2 boards are required.

- Atmel SAMV71 Flamenco Host Board

- Flamenco Breakout Board

Figure 1 Atmel SAMV71 Flamenco Host Board

Figure 2 Flamenco Breakout Board

2.2. Jumper configuration

Initial jumper configuration is for the host board is as follow:

| JP1 | MCU |
| --- | --- |
| JP2 | EDBG |
| JP3 | VCC (3V3) |

Change JP2 setting when logging to a terminal program on PC as follow:

| JP2 | FTDI |
| --- | --- |

Figure 3 Initial Jumper Configuration

2.3. Board connection

Initially, connect the breakout board to the host board and plug USB micro type-B cable to EDBG USB Port(J300) on the host board.

Figure 4 Initial Boards Connection

After flashing the host board via EDBG port, when logging to a terminal program, plug USB micro type-B cable to FTDI USB Port(J2) on the host board. Also, change JP2 jumper setting as described in the section 2.2.

Figure 5 Connection when logging to a terminal

2.4. PPS interface

To enable PPS interface, provide an external 1Hz pulse signal to the center pin of JP1 jumper.

Figure 6 PPS interface

- SW Environment

3.1. Prerequisite

To build and use an example application provided as part for the driver SW package, the following 3rd party software is required:

- Atmel Studio 7:  https://www.microchip.com/mplab/avr-support/atmel-studio-7

- FTDI drivers to retrieve data from FTDI port: https://ftdichip.com/drivers/vcp-drivers/

For the serial communication between PC and the host board, a terminal program on PC (such as PuTTY) is required.

3.2. SW package description

The name of package file is as follow:

- IIM4623x_(version).zip

The following file in the package has all source code for IIM4623x driver:

- IIM4623x\IIM4623x\src\iim4623x\iim4623x.c

The following file in the package contains firmware image for IIM4623x (the version of firmware is described in IIM4623x MCU Driver Software Release Notes):

- IIM4623x\IIM4623x\src\iim4623x\iim4623x_fw_img_(FW version).h

The following file in the package has all source code which is dependent on Atmel platform:

- IIM4623x\IIM4623x\src\system.c

The following file in the package has all source code for example application:

- IIM4623x\IIM4623x\src\main.c

3.3. Configuration of the Interface between host board and breakout board

In ‘main.c’ file, there are macro definitions to decide which interface between host board and breakout board will be used.

If you want to use UART, you can set macro as follows.

#define SERIF_TYPE_SPI	0

#define SERIF_TYPE_UART	1

If you want to use SPI, you can set macro as follows.

#define SERIF_TYPE_SPI	1

#define SERIF_TYPE_UART	0

3.4. Building & running example application

Follow steps below to build and run example application.

1. Connect the host board to PC using USB micro type-B cable. Check if ‘EDBG Virtual COM Port’ exists at the Device Manager of PC.

2. Double-click the Atmel Studio 7.0 Solution File in the package (IIM4623x\IIM4623x.atsln).

3. Check if ‘EDBG’ is selected at the Tool section of ‘Atmel Studio -> Project -> Properties’ menu.

Figure 7 Select debugger at the Tool section

4. Set the UART baud rate of a terminal program on PC is ‘3000000’.

5. Select the menu ‘Atmel Studio -> Debug -> Start Without Debugging’. Then Atmel Studio starts building and downloading. After finishes downloading, the example application starts running.

3.5. Key input description

Once the example application starts running, it shows the following menu texts.

Figure 8 Menu of the example application

Then, if we type a character showed on the main menu, a command will be sent to IIM4623x and the corresponding function will be executed. If we type ‘s’ on the terminal window, ‘start streaming’ command will be sent to IIM4623x and then it will start Accel, Gyro and Temperature data streaming at 1kHz by default. Before starting streaming, we can configure ODR of streaming, output data form and LPF BW etc. by typing the corresponding character showed on the menu. Some items in the main menu can show their sub-menu when typing key input for them. IIM4623x supports two kinds of operation mode. One is Command Mode and the other is Streaming Mode. In Streaming Mode, ‘stop streaming’ and ‘set UTC time’ commands can be sent to IIM4623x. All commands except ‘stop streaming’ can be used in Command Mode.

3.6. Streaming data description

After selecting Data Output and starting streaming, streaming data are shown on the console window following the manner below.

1. The overall order of contents is sample counter, timestamp, sensor data, and sensor status.

2. A comma separates each content value.

3. Timestamp is generated by IIM4623x.

4. If you select all sensor data outputs via ‘select Data Output’ menu, sensor data are shown on the console window following the order below.

- Accel (X, Y, Z), Gyro (X, Y, Z), Temperature, Delta Velocity (X, Y, Z), Delta Angle (X, Y, Z)

Figure 9 Streaming data

3.7. Configurations that need to be set before performing actions

1. Before setting UTC time, ‘Sync with PPS’ need to be enabled via ‘select Sync Config’ menu.

2. To start data streaming, at least one data output needs to be enabled via ‘select Data Output’ menu.

- Revision History

| Driver Version | Date | Description | Author |
| --- | --- | --- | --- |
| v0.1 | Feb 19, 2020 | Initial version. | TDK Invensense JW Yoon |
| v0.5 | March 3, 2020 | Update SW package description and Key input description. | JW Yoon |
| v1.1 | April 1, 2020 | Change the description on setting the UART baud rate of a terminal program. | JW Yoon |
| v1.2 | May 1, 2020 | Support IIM46230. Add a description on configuration of the Interface between host board and breakout board. | JW Yoon |
| v1.3 | May 21, 2020 | Change Figure 6 and the description on commands which can be sent in streaming mode. | JW Yoon |
| v1.4 | June 12, 2020 | Add the sections 2.4, 3.6 and 3.7. | JW Yoon |
| v2.0 | July 8, 2022 | Add description on firmware image file. Remove description on setting bias calibration in the section 3.7. | JW Yoon |
| v2.1 | July 13, 2022 | Change data logging format to keep backward compatibility with the old tool. | JW Yoon |
| v2.2 | Aug 2, 2022 | Add description on support for FTDI port. | JW Yoon |